Spatial and Distance Metrics

- Total Path Length: This represents the total cumulative distance the CoP travels over the entire recording period(Already Tracked).

- Directional Path Lengths: You can split the total CoP path into independent Medial-Lateral (side-to-side) and Anterior-Posterior (front-to-back) path lengths.

- Sway Range: This is calculated by finding the maximum distance between the furthest points of sway in both the ML and AP directions.

- Root Mean Square (RMS) of Displacement: The RMS of the CoP displacement in both the ML and AP directions is                         a standard measure of sway magnitude. You can calculate this using the formula ​, where di​ is the displacement at each data point.

- Average Radial Position: This metric measures the average distance of the CoP from the mean center point throughout the trial.

- Maximum Radial Position: This identifies the single furthest point the CoP reaches from the mean center during the recording.

Area Metrics

- Swept Area: This calculates the total area covered by the movement of the CoP.

- Area of the Confidence Ellipse: This fits an estimating ellipse that encompasses the CoP data points, providing a metric of the overall area of postural sway(Already tracked).

Pressure Distribution and Sensor Features

- Contact Area Variations: By setting a predefined force threshold, you can determine which areas of the board are actively bearing weight and how that dynamic footprint changes based on the sitting or standing position.

- Regional Pressure Ratios: You can calculate the ratio of pressure between different regional contact areas to explicitly capture leaning or postural changes.

- Centroids: Similar to the overall CoP, you can calculate the mass-weighted average of the pressure points to find localized centers of mass along the x and y axes.

- Histogram of Oriented Gradients (HOG): If you treat the pressure data over time as an image or matrix, you can apply HOG analysis to extract patterns in how the pressure gradients shift.